Update Sapphire_WUXGA  Projector FW SOP

- Setup projector in F/W downloading mode

1. Press [Power] + [INPUT] simultaneously

2.Plug-in power code

3. Keep press [power + INPUT] button until the power LED in flashing light

4. Connect a Micro_ USB cable to desktop/laptop

- Start updating F/W

1. Launch FlashUpgrader(pw868_use) from folder

2. Select USB2.0 and press Flash

3. It will start to upgrade and just wait a few minutes

Projector will turn on after updating finished.

5. Unplug Micro_USB and Power cord

6. Done.